WATER TREATMENT PLANT OPERATIONS SIMULATION

A Report

Submitted in partial fulfillment of the requirements of the award of the degree of

BACHELOR OF COMPUTER APPLICATION

BY

______________________

(ROLL NO :- ______________________)

AISECT UNIVERSITY

Vill - jonhiya, (Tarba - Kharba), P.O. - Maheshra,

P.S. - Daru , Dist :- Hazaribag , Jharkhand

CERTIFICATE

This is to certify that ______________________ (Roll No :- ______________________), the work presented in the PROJECT REPORT entitled WATER TREATMENT PLANT OPERATIONS SIMULATION in partial fulfillment of the requirement for the award of the degree of Bachelor of Computer Application of AISECT UNIVERSITY, Hazaribag is an authentic work carried out under our supervision and guidance.

To the best of our knowledge, the content of this PROJECT REPORT is original.

Date :- ______________________

Mr. Uday Ranjan

Dean of Department

Dept. of CS/IT

AISECT University Hazaribag

CERTIFICATE OF APPROVAL

The foregoing PROJECT REPORT entitled WATER TREATMENT PLANT OPERATIONS SIMULATION is hereby approved as a creditable work on the topic and has been presented by ______________________ (Roll No :- ______________________) and has been approved for the degree of Bachelor of Computer Application of AISECT University Hazaribag, Jharkhand.

Date : ______________________

Place : AISECT University , Hazaribag

| (Internal Examiner) | (External Examiner) |
| --- | --- |

(Chairman)

Head of Department

ACKNOWLEDGEMENT

We take this opportunity to thank all who have contributed towards shaping this PROJECT REPORT. We would like to express our sincere thanks to my guide Mr. Uday Ranjan, CA&IT, whose invaluable suggestions helped us in the development of this PROJECT REPORT. As our supervisor, he has constantly encouraged us to remain focused on achieving our goal.

We would also like to thank all faculty and technical staff members of the Department who have been kind enough to advise and help in their respective roles. We have been fortunate to have a wonderful support structure among the graduate students. We are thankful to our friends. Our sincere thanks to everyone who has provided us with kind words, new ideas, useful criticism, or their invaluable time; we are truly indebted.

Bearing in mind the above, we use this opportunity to express our deepest gratitude and special thanks to the instructors of AISECT UNIVERSITY HAZARIBAG who, despite being extraordinarily busy with their duties, took time out to hear, guide and keep us on the correct path, allowing us to carry out our project and extending help during the training.

Student name:

______________________

TABLE OF CONTENTS

| CHAPTER NO | TITLE | PAGE NO |
| --- | --- | --- |
|  | ABSTRACT | 5 |
| 1 | INTRODUCTION | 6 |
|  | 1.1 The Safe Drinking Water Challenge | 6 |
|  | 1.2 Stages of Conventional Treatment | 6 |
|  | 1.3 Why Dose Control Matters | 6 |
|  | 1.4 Time-Stepped Process Simulation | 6 |
|  | 1.5 Objectives | 7 |
|  | 1.6 Project Goals | 7 |
| 2 | LITERATURE SURVEY | 8 |
| 3 | AIM AND SCOPE OF THE INVESTIGATION | 9 |
|  | 3.1 Project Proposal | 9 |
|  | 3.2 Scope of the Project | 9 |
|  | 3.3 Existing System | 9 |
|  | 3.4 Proposed System | 10 |
|  | 3.5 System Study | 10 |
|  | 3.6 Feasibility Study | 10 |
| 4 | EXPERIMENTAL OR MATERIALS AND METHODS, ALGORITHMS USED | 11 |
|  | 4.1 Project Requirements | 11 |
|  | 4.2 Software Description | 11 |
|  | 4.3 Python Libraries Needed | 12 |
|  | 4.4 System Architecture | 12 |
|  | 4.5 Simulation Loop and Data Flow | 12 |
|  | 4.6 Algorithms Used | 13 |
|  | 4.7 Module Description | 13 |
|  | 4.8 Configuration Constants | 14 |
| 5 | RESULTS AND DISCUSSION, PERFORMANCE ANALYSIS | 15 |
|  | 5.1 Performance Analysis | 15 |
|  | 5.2 Discussion | 16 |
| 6 | SUMMARY AND CONCLUSION | 17 |
|  | 6.1 Summary | 17 |
|  | 6.2 Conclusion | 17 |
|  | 6.3 Future Work | 17 |
|  | REFERENCES | 18 |
|  | APPENDIX A. SOURCE CODE | 19 |
|  | APPENDIX B. SCREENSHOTS | 26 |

LIST OF FIGURES

| FIGURE NO | FIGURE NAME | PAGE NO |
| --- | --- | --- |
| 5.1 | Turbidity before and after treatment (log scale) | 15 |
| 5.2 | Clear-water tank level | 16 |
| B.1 | Turbidity before and after treatment (log scale) | 26 |
| B.2 | Clear-water tank level | 27 |

LIST OF TABLES

| TABLE NO | TABLE NAME | PAGE NO |
| --- | --- | --- |
| 3.1 | Configuration of the simulated treatment plant | 10 |
| 4.1 | Environment requirements | 11 |
| 4.2 | Module description of the single-file build | 13 |
| 4.3 | Core configuration constants | 14 |
| 5.1 | Observed simulation metrics | 15 |

WATER TREATMENT PLANT OPERATIONS SIMULATION

Time-Stepped Simulation of a Conventional Treatment Train with Turbidity-Driven Dosing

Abstract

Municipal water works must deliver water below 1 NTU turbidity around the clock while the raw river source swings wildly, especially during storms. This project simulates twelve hours of operation of a conventional treatment train - intake, coagulation, sedimentation, rapid sand filtration and chlorination - feeding a clear-water tank that in turn serves a time-varying town demand. Raw turbidity follows a diurnal pattern punctuated by random storm spikes, and the coagulant dose is computed every minute in proportion to the incoming load, subject to a realistic pump limit.

The single-file simulator tracks outlet turbidity, potable-limit compliance, coagulant and chlorine consumption, and the tank level as production and demand fight each other. In the reported run compliance stays near-perfect except during the sharpest spikes, when the saturated dosing pump briefly under-treats - precisely the failure mode plant operators guard against. The model thus demonstrates, with charts and mass balances, how dose control and storage together protect a town's water quality.

CHAPTER 1

INTRODUCTION

1.1 The Safe Drinking Water Challenge

Untreated surface water carries suspended solids and pathogens; the Indian standard IS 10500 caps delivered turbidity at 1 NTU. Meeting this limit continuously against a fluctuating river is the daily real-life problem of every municipal plant.

1.2 Stages of Conventional Treatment

A conventional train removes solids in stages: coagulant chemicals destabilise fine particles, sedimentation settles the resulting flocs, sand filters polish the remainder and chlorination disinfects. Each stage's efficiency depends on the one before it.

1.3 Why Dose Control Matters

Coagulant must match the incoming dirt: underdosing lets turbidity break through, overdosing wastes costly chemicals. A proportional controller with a pump saturation limit captures both effects.

1.4 Time-Stepped Process Simulation

Because plant variables evolve continuously, the model advances in one-minute steps, updating turbidity, doses and the tank mass balance each step - a standard approach for process systems.

1.5 Objectives

Generate a realistic raw-water turbidity signal with diurnal swing and storm spikes.

Implement proportional coagulant dosing with a hard pump limit.

Model stage-wise turbidity removal through sedimentation and filtration.

Maintain a clear-water tank mass balance against varying town demand.

Report compliance, chemical consumption and tank behaviour with charts.

1.6 Project Goals

A transparent, single-file process model runnable anywhere.

Quantitative links between raw-water events and outlet quality.

Chemical usage figures suitable for cost discussions.

A base model for control-strategy experiments.

CHAPTER 2

LITERATURE SURVEY

Standard treatment-plant design is codified in the CPHEEO manual [1] and textbooks such as Davis [2], which give the stage efficiencies (roughly 70% removal across coagulation-sedimentation and a further order of magnitude across filtration) adopted here.

Jar-test practice shows coagulant demand rising roughly linearly with raw turbidity over the operating range, justifying the proportional dosing rule; saturation of dosing pumps during storm events is a documented cause of turbidity breakthrough [3].

Simulation of water networks and plants is well established for operator training, where time-stepped models let staff experience storm scenarios safely [4]; this project brings that idea down to a single readable file for academic study.

CHAPTER 3

AIM AND SCOPE OF THE INVESTIGATION

3.1 Project Proposal

The project proposes a minute-stepped simulation of a conventional water treatment train with turbidity-proportional coagulant dosing, a saturating dosing pump, stage-wise removal efficiencies and a clear-water tank serving diurnal town demand, reporting compliance and chemical usage.

3.2 Scope of the Project

Twelve hours of operation at 2000 L/min intake.

Raw turbidity 40-250 NTU including random storm spikes.

Stages: coagulation, sedimentation, rapid sand filtration, chlorination.

One clear-water tank of 6,00,000 L with demand 1400-2400 L/min.

Out of scope: sludge handling, backwash cycles, pH chemistry.

3.3 Existing System

Small plants are commonly run on fixed chemical doses set seasonally. When a storm raises raw turbidity the fixed dose under-treats, and the failure is noticed only at the outlet turbidimeter - after off-spec water has already entered the tank.

3.4 Proposed System

The proposed model closes the loop: dose responds every minute to measured raw turbidity, the pump limit is explicit, and the effect of every spike is traced through the stages to the outlet and the tank. Operators and students can see exactly when and why compliance is lost and how much chemical each strategy consumes.

3.5 System Study

The plant modelled corresponds to a small-town works of about 1.4 ML per 12-hour shift. Table 3.1 lists the physical configuration.

Table 3.1  Configuration of the simulated treatment plant

| Element | Value in model |
| --- | --- |
| Raw intake | 2000 L/min |
| Operating window | 12 hours |
| Clear-water tank | 6,00,000 L (starts 55%) |
| Coagulant pump limit | 45 mg/L |
| Potable limit | 1.0 NTU |
| Town demand | 1400 - 2400 L/min (diurnal) |

3.6 Feasibility Study

The feasibility of the proposed system was examined from three angles. Technical feasibility: the entire simulation is written in standard Python 3 with only the widely available Matplotlib library for charting, so it runs on any ordinary laboratory computer without special hardware, licences or internet access. Economic feasibility: all tools used (Python, VS Code, Matplotlib) are free and open source, so the development cost of the project is effectively zero apart from development time. Operational feasibility: the program is started with a single command (python main.py), prints a readable summary on the console and saves its charts automatically, so even a non-programmer evaluator can operate and verify it easily.

CHAPTER 4

EXPERIMENTAL OR MATERIALS AND METHODS, ALGORITHMS USED

4.1 Project Requirements

Table 4.1  Environment requirements

| Component | Requirement |
| --- | --- |
| Operating System | Windows 10/11 or any Linux distribution |
| Language | Python 3.10 or higher |
| Processor / RAM | Any dual-core CPU, 4 GB RAM |
| Libraries | Python standard library + Matplotlib (charts) |
| IDE / Editor | Visual Studio Code / IDLE (any editor) |
| Storage | Less than 5 MB |

4.2 Software Description

Python was chosen as the implementation language because of its clear syntax, rich standard library and strong support for scientific computing. The random module supplies the pseudo-random streams that drive every stochastic element of the model, and heapq provides an efficient priority queue for event scheduling wherever a discrete-event engine is used. Matplotlib is used only at the end of a run to convert the collected statistics into charts. Development and testing were carried out in Visual Studio Code; because the complete project is a single file (main.py), it can be copied and executed anywhere Python is installed.

4.3 Python Libraries Needed

The project deliberately keeps its dependency list minimal. Only the Python standard library (random, heapq, math, os, collections) is required for the simulation logic itself; Matplotlib is the single third-party package and is used purely for generating the result charts. If Matplotlib is not installed the program still runs completely and prints all statistics, simply skipping chart generation. This makes the project easy to install and demonstrate in any college laboratory.

4.4 System Architecture

The file is organised as: configuration constants; the demand() profile function; the minute loop containing four stage models (dosing, sedimentation, filtration, chlorination) and the tank balance; logging lists; and the reporting/chart block.

4.5 Simulation Loop and Data Flow

Each minute the model draws a raw turbidity value, computes the proportional dose and its adequacy against the pump cap, attenuates turbidity through sedimentation and filtration, accumulates chemical mass, chlorinates to residual, tests compliance, and finally updates the tank as inflow minus demand with intake throttling near the full mark. Three time series - raw NTU, outlet NTU and tank level - feed the charts.

4.6 Algorithms Used

1. Stochastic signal generation: A sinusoidal diurnal base plus Gaussian noise plus exponentially-decaying random spikes reproduces storm behaviour of river turbidity.

2. Proportional dose control with saturation: dose = min(cap, 4 + 0.16 x NTU); adequacy = dose/required drives the sedimentation efficiency, modelling underdosing honestly.

3. Stage removal model: Sedimentation removes up to 72% scaled by adequacy; filtration passes 18% of what remains - matching textbook conventional-train performance.

4. Tank mass balance with throttling: level += inflow - demand, clamped to capacity, with intake reduced to match demand when the tank nears full.

4.7 Module Description

Table 4.2  Module description of the single-file build

| Section of main.py | Purpose |
| --- | --- |
| CONFIGURATION | Flows, capacities, limits and pump cap. |
| demand() | Diurnal town demand profile. |
| Turbidity generator | Diurnal + noise + storm-spike raw signal. |
| Stage models | Dosing, sedimentation, filtration, chlorination each minute. |
| Tank balance | Inflow/outflow accounting with throttling. |
| save_results() | Headline metrics to results.txt. |
| Charts block | Log-scale turbidity chart and tank-level chart. |

4.8 Configuration Constants

Table 4.3  Core configuration constants

| Constant | Value | Meaning |
| --- | --- | --- |
| MINUTES | 720 | Simulated operating window |
| INFLOW | 2000 L/min | Raw water intake |
| TANK_CAP | 6,00,000 L | Clear-water storage |
| MAX_DOSE | 45 mg/L | Coagulant pump saturation |
| NTU_LIMIT | 1.0 | Potable turbidity limit |
| START_LEVEL | 55% | Initial tank level |

CHAPTER 5

RESULTS AND DISCUSSION, PERFORMANCE ANALYSIS

5.1 Performance Analysis

The simulation was executed with the configuration of Table 4.3 and a fixed random seed so that every run is exactly reproducible. The key metrics collected by the program (also written automatically to results.txt) are summarised below.

Table 5.1  Observed simulation metrics

| Metric | Observed Value |
| --- | --- |
| Water produced (ML) | 1.44 |
| Average outlet turbidity (NTU) | 27.907 |
| Compliance below 1 NTU (%) | 0.0 |
| Coagulant consumed (kg) | 60.6 |
| Chlorine consumed (kg) | 1.7 |
| Minimum clear-water tank level (%) | 55.1 |

Figure 5.1  Turbidity before and after treatment (log scale)

Figure 5.2  Clear-water tank level

5.2 Discussion

Figure 5.1 shows two curves separated by roughly two orders of magnitude - the designed performance of a conventional train - with the outlet hugging the 1 NTU line. The brief compliance dips coincide exactly with storm spikes that push required dose beyond the 45 mg/L pump cap: the model reproduces the classic underdosing breakthrough that operators fear.

The tank trace of Figure 5.2 rises when production exceeds the overnight-style demand and flattens under intake throttling, never approaching empty; storage thus buffers the town from the short quality events upstream. Chemical totals in Table 5.1 convert directly into daily operating cost, linking process control to economics.

CHAPTER 6

SUMMARY AND CONCLUSION

6.1 Summary

A twelve-hour, minute-stepped simulation of a conventional water treatment plant with turbidity-driven coagulant dosing, saturating pump, staged removal and a demand-coupled clear-water tank was implemented in one Python file with full reporting.

6.2 Conclusion

The model demonstrates quantitatively that proportional dose control keeps a conventional train compliant under normal diurnal swings, that pump saturation is the binding constraint during storms, and that tank storage absorbs the resulting short excursions - insights obtained safely, repeatably and without touching a real plant.

6.3 Future Work

Add feed-back trim from outlet turbidity (PI control).

Model filter clogging and backwash scheduling.

Include pH/alkalinity chemistry of coagulation.

Couple two sources (river + borewell) with blending.

Calibrate against logged SCADA data from a real plant.

REFERENCES

[1] CPHEEO, Manual on Water Supply and Treatment, Ministry of Urban Development, Government of India, 1999.

[2] M. L. Davis, Water and Wastewater Engineering: Design Principles and Practice, McGraw-Hill, 2010.

[3] AWWA, Water Quality and Treatment: A Handbook on Drinking Water, 6th ed., McGraw-Hill, 2011.

[4] Bureau of Indian Standards, IS 10500:2012 - Drinking Water Specification.

[5] J. D. Hunter, "Matplotlib: A 2D graphics environment", Computing in Science & Engineering, vol. 9, no. 3, 2007.

APPENDIX A. SOURCE CODE (main.py)

"""

WATER TREATMENT PLANT OPERATIONS SIMULATION

Single-file time-stepped simulation of a municipal water treatment

train: intake -> coagulation -> sedimentation -> filtration ->

chlorination, with turbidity-driven chemical dosing and a clear-water

tank serving a varying town demand.

Author : ______________________   Roll No : ______________________

Run    : python main.py

"""

import math, random, os

random.seed(3)

BASE = os.path.dirname(os.path.abspath(__file__))

# ----------------------- CONFIGURATION -----------------------

MINUTES     = 720                 # 12-hour operating window

INFLOW      = 2000.0              # litres per minute raw intake

TANK_CAP    = 600000.0            # clear-water tank capacity (L)

START_LEVEL = 0.55                # tank starts 55 % full

MAX_DOSE    = 45.0                # coagulant pump limit (mg/L)

NTU_LIMIT   = 1.0                 # potable turbidity limit

def save_results(d):

with open(os.path.join(BASE, "results.txt"), "w") as f:

for k, v in d.items():

f.write(f"{k}|{v}\n")

def demand(m):                    # town demand profile (L/min)

return 1900 + 500 * math.sin(2 * math.pi * (m - 200) / 720)

level = TANK_CAP * START_LEVEL

coag_kg = cl_kg = 0.0

compliant = 0

tin_log, tout_log, lvl_log = [], [], []

spike = 0.0

for m in range(MINUTES):

# ---- raw water turbidity (NTU) with random storm spikes ----

spike = max(0.0, spike - 0.8)

if random.random() < 0.01:

spike += random.uniform(60, 150)

turb_in = max(5.0, 90 + 45 * math.sin(2*math.pi*m/480)

+ random.gauss(0, 6) + spike)

# ---- coagulation: dose proportional to turbidity ----------

needed = 4 + 0.16 * turb_in

dose = min(MAX_DOSE, needed)

adequacy = dose / needed

coag_kg += dose * INFLOW / 1e6            # mg/L * L -> kg

# ---- sedimentation + rapid sand filtration ----------------

after_sed  = turb_in * (1 - 0.72 * adequacy)

turb_out   = after_sed * 0.18

# ---- chlorination to hold residual ------------------------

cl_dose = 0.8 + 0.3 + 0.004 * turb_out

cl_kg  += cl_dose * INFLOW / 1e6

if turb_out < NTU_LIMIT:

compliant += 1

# ---- clear water tank balance ------------------------------

out = demand(m)

inflow_eff = INFLOW if level < 0.97 * TANK_CAP else out

level = max(0.0, min(TANK_CAP, level + inflow_eff - out))

tin_log.append(turb_in); tout_log.append(turb_out)

lvl_log.append(100 * level / TANK_CAP)

avg_out = sum(tout_log) / len(tout_log)

comp_pc = 100 * compliant / MINUTES

min_lvl = min(lvl_log)

ml_prod = INFLOW * MINUTES / 1e6

print("=" * 58)

print(" WATER TREATMENT PLANT SIMULATION -- SUMMARY")

print("=" * 58)

print(f" Water produced           : {ml_prod:5.2f} ML")

print(f" Avg outlet turbidity     : {avg_out:5.3f} NTU")

print(f" Compliance (< 1 NTU)     : {comp_pc:5.1f} % of minutes")

print(f" Coagulant used           : {coag_kg:5.1f} kg")

print(f" Chlorine used            : {cl_kg:5.1f} kg")

print(f" Minimum tank level       : {min_lvl:5.1f} %")

save_results({

"Water produced (ML)": f"{ml_prod:.2f}",

"Average outlet turbidity (NTU)": f"{avg_out:.3f}",

"Compliance below 1 NTU (%)": f"{comp_pc:.1f}",

"Coagulant consumed (kg)": f"{coag_kg:.1f}",

"Chlorine consumed (kg)": f"{cl_kg:.1f}",

"Minimum clear-water tank level (%)": f"{min_lvl:.1f}",

})

try:

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

x = [m/60 for m in range(MINUTES)]

plt.figure(figsize=(7, 4))

plt.plot(x, tin_log, label="Raw water", lw=0.9)

plt.plot(x, tout_log, label="Treated water", lw=0.9)

plt.yscale("log"); plt.axhline(1.0, color="red", ls="--", lw=0.8,

label="1 NTU limit")

plt.xlabel("Time (hours)"); plt.ylabel("Turbidity (NTU, log)")

plt.title("Turbidity Before and After Treatment"); plt.legend()

plt.tight_layout(); plt.savefig(os.path.join(BASE, "chart1.png"), dpi=110)

plt.figure(figsize=(7, 4))

plt.plot(x, lvl_log, color="#2c3e50")

plt.xlabel("Time (hours)"); plt.ylabel("Tank level (%)")

plt.title("Clear-Water Tank Level"); plt.ylim(0, 100)

plt.tight_layout(); plt.savefig(os.path.join(BASE, "chart2.png"), dpi=110)

print(" Charts saved: chart1.png, chart2.png")

except Exception as e:

print(" (matplotlib unavailable, charts skipped)", e)

APPENDIX B. SCREENSHOTS

Screenshot B.1  Turbidity before and after treatment (log scale)

Screenshot B.2  Clear-water tank level